UART with Baud Rate Generator

RTL Design & Verification — Complete Notes

# 1. What is UART?

UART (Universal Asynchronous Receiver/Transmitter) is a point-to-point serial communication protocol with no shared clock between sender and receiver. Both sides agree on a baud rate in advance and each derives its own timing.

Because there is no clock line, the receiver must continuously sample the line to detect when data arrives. This is why the protocol is called asynchronous.

## 1.1 Frame Format (8N1)

The most common UART configuration is 8N1: 8 data bits, No parity, 1 stop bit. Each frame looks like:

Idle:      line held HIGH (logic 1) — default resting state

Start bit: line pulled LOW (logic 0) — signals frame start

D0–D7:     8 data bits, transmitted LSB first

Stop bit:  line HIGH (logic 1) — at least 1 bit period

Total:     10 bit-periods per frame for 8N1

The start bit is the key: the 1→0 transition wakes the receiver and tells it to begin sampling. The stop bit confirms the frame ended cleanly — if it reads 0 instead, that is a framing error.

## 1.2 Baud Rate

Baud rate = number of symbols (bits) per second. Common values: 9600, 19200, 38400, 57600, 115200. Higher baud rates require more accurate clocking.

One bit period in time = 1 / baud_rate seconds. At 115200 baud: 1/115200 ≈ 8.68 µs per bit.

# 2. Baud Rate Generator

## 2.1 Why we need it

The FPGA system clock (e.g. 50 MHz) is far faster than the baud rate. The baud rate generator divides the system clock down to a timing reference for UART. We use 16× oversampling: the generator produces 16 ticks per bit period. This lets the receiver sample each bit at its center (after 8 ticks) rather than at the edge, giving noise immunity.

## 2.2 The Math

| Parameter | Formula | Example (50MHz / 115200) |
| --- | --- | --- |
| Tick frequency | BAUD × 16 | 1,843,200 Hz |
| DIVISOR | CLK_FREQ ÷ (BAUD × 16) | 50,000,000 ÷ 1,843,200 = 27.1 → 27 |
| Baud error | ¦(actual − target)¦ ÷ target | ~0.4% (within 2–3% tolerance) |

## 2.3 RTL Code

module baud_rate_gen #(

parameter CLK_FREQ  = 50_000_000,

parameter BAUD_RATE = 115200

)(

input  wire clk,

input  wire rst_n,

output reg  tick

);

localparam DIVISOR = CLK_FREQ / (BAUD_RATE * 16);  // = 27

reg [$clog2(DIVISOR)-1:0] count;

always @(posedge clk or negedge rst_n) begin

if (!rst_n) begin

count <= 0; tick <= 0;

end else begin

if (count == DIVISOR - 1) begin

count <= 0; tick <= 1;

end else begin

count <= count + 1; tick <= 0;

end

end

end

endmodule

## 2.4 Key Points

- localparam DIVISOR computed at elaboration — no magic numbers

- $clog2(DIVISOR) auto-sizes the counter register — synthesizer best practice

- tick is a single-cycle pulse, not a level signal

- Asynchronous active-low reset (negedge rst_n in sensitivity list)

# 3. UART Transmitter (TX)

## 3.1 FSM States

| State | tx_out value | Duration | Transition |
| --- | --- | --- | --- |
| IDLE | 1 (high) | Until tx_start | → START on tx_start |
| START | 0 (low) | 16 ticks (1 bit) | → DATA |
| DATA | shift_reg[0] | 16 ticks × 8 bits | → STOP after bit 7 |
| STOP | 1 (high) | 16 ticks (1 bit) | → IDLE |

## 3.2 Design Decisions

- tick_cnt counts 0–15; state advances only when tick_cnt reaches 15 — this implements the 1 bit = 16 ticks relationship

- Data is latched into shift_reg on tx_start, so caller can change tx_data immediately after

- shift_reg >> 1 shifts next bit into [0] each bit period — cleaner than indexing with bit_idx

- tx_busy held high throughout transmission so caller knows not to assert tx_start again

## 3.3 RTL Code

module uart_tx (

input  wire       clk, rst_n, tick, tx_start,

input  wire [7:0] tx_data,

output reg        tx_out, tx_busy

);

localparam IDLE=2'd0, START=2'd1, DATA=2'd2, STOP=2'd3;

reg [1:0] state;

reg [3:0] tick_cnt;

reg [2:0] bit_idx;

reg [7:0] shift_reg;

always @(posedge clk or negedge rst_n) begin

if (!rst_n) begin

state<=IDLE; tx_out<=1; tx_busy<=0;

tick_cnt<=0; bit_idx<=0; shift_reg<=0;

end else case(state)

IDLE: begin

tx_out<=1; tx_busy<=0;

if (tx_start) begin

shift_reg<=tx_data; tick_cnt<=0;

state<=START; tx_busy<=1;

end

end

START: begin

tx_out<=0;

if (tick) begin

if (tick_cnt==15) begin tick_cnt<=0; bit_idx<=0; state<=DATA; end

else tick_cnt<=tick_cnt+1;

end

end

DATA: begin

tx_out<=shift_reg[0];

if (tick) begin

if (tick_cnt==15) begin

tick_cnt<=0; shift_reg<=shift_reg>>1;

if (bit_idx==7) state<=STOP;

else bit_idx<=bit_idx+1;

end else tick_cnt<=tick_cnt+1;

end

end

STOP: begin

tx_out<=1;

if (tick) begin

if (tick_cnt==15) begin

tick_cnt<=0; state<=IDLE; tx_busy<=0;

end else tick_cnt<=tick_cnt+1;

end

end

endcase

end

endmodule

# 4. UART Receiver (RX)

## 4.1 The Oversampling Trick

When the RX line drops to 0 (start bit detected), we do NOT sample immediately — we wait 8 ticks to land at the center of the start bit. From there, every subsequent sample is taken 16 ticks later — always at the bit center. This is the core of 16× oversampling.

Centering the sample point maximises the margin against clock drift and signal edges. A UART tolerates up to ~2–3% baud rate error per frame thanks to this technique.

## 4.2 Glitch Rejection

After detecting the falling edge, the RX waits 8 ticks and checks the line again. If it has gone back to 1, the falling edge was a noise glitch and we return to IDLE without processing a frame. Real noise spikes are shorter than 8 tick periods (4× baud period).

## 4.3 Shift Register Direction

UART sends LSB first. The RX shift register uses:

shift_reg <= {rx_in, shift_reg[7:1]};

This shifts rx_in into the MSB each time. After 8 bits, the first received bit (D0) has shifted all the way to position [0], which is correct. The LSB-first convention reverses itself naturally.

## 4.4 RTL Code

module uart_rx (

input  wire       clk, rst_n, tick, rx_in,

output reg  [7:0] rx_data,

output reg        rx_valid

);

localparam IDLE=2'd0, START=2'd1, DATA=2'd2, STOP=2'd3;

reg [1:0] state;  reg [3:0] tick_cnt;

reg [2:0] bit_idx; reg [7:0] shift_reg;

reg rx_prev;

wire start_detected = rx_prev & ~rx_in;  // falling edge

always @(posedge clk or negedge rst_n)

if (!rst_n) rx_prev<=1; else rx_prev<=rx_in;

always @(posedge clk or negedge rst_n) begin

if (!rst_n) begin

state<=IDLE; tick_cnt<=0; bit_idx<=0;

shift_reg<=0; rx_data<=0; rx_valid<=0;

end else begin

rx_valid<=0;  // default: pulse only 1 cycle

case(state)

IDLE: if (start_detected) begin tick_cnt<=0; state<=START; end

START: if (tick) begin

if (tick_cnt==7) begin

if (rx_in==0) begin tick_cnt<=0; bit_idx<=0; state<=DATA; end

else state<=IDLE;  // glitch — abort

end else tick_cnt<=tick_cnt+1;

end

DATA: if (tick) begin

if (tick_cnt==15) begin

shift_reg<={rx_in,shift_reg[7:1]};

tick_cnt<=0;

if (bit_idx==7) state<=STOP;

else bit_idx<=bit_idx+1;

end else tick_cnt<=tick_cnt+1;

end

STOP: if (tick) begin

if (tick_cnt==15) begin

if (rx_in==1) begin

rx_data<=shift_reg; rx_valid<=1;

end

state<=IDLE; tick_cnt<=0;

end else tick_cnt<=tick_cnt+1;

end

endcase

end

end

endmodule

# 5. Testbench

## 5.1 Loopback Test Strategy

The simplest verification: connect tx_out directly to rx_in. Send a known byte (0xA5), wait for rx_valid, compare rx_data to 0xA5. This tests the full TX→RX pipeline without external hardware.

## 5.2 Key Testbench Constructs

| Construct | Purpose |
| --- | --- |
| `timescale 1ns/1ps | Set simulation time unit and precision |
| always #10 clk=~clk | Generate 50MHz clock (20ns period) |
| @(posedge rx_valid) | Block until rx_valid rises — cleaner than polling |
| wait(!tx_busy) | Wait for TX to finish before checking RX |
| $dumpfile / $dumpvars | Write VCD waveform for GTKWave viewing |
| $display / $finish | Print result and end simulation |
| Initial timeout block | Watchdog — prevents infinite hang on bugs |

## 5.3 Common Bugs to Watch For

- tx_start held high for more than 1 cycle: TX will try to send multiple frames

- rx_data sampled before rx_valid: data is not yet valid, will read stale/zero

- Forgetting to deassert reset before stimulus: FSMs stay stuck in reset

- Loopback missing: rx_in left unconnected — receiver sees random logic level

- Timeout too short: for 115200 baud in 50MHz sim, one frame takes ~435 µs → ~21,700 clock cycles

# 6. Verilog Quick Reference

## 6.1 Assignments

| Type | Syntax | Use when |
| --- | --- | --- |
| Non-blocking | a <= b | Inside clocked always block (registers) |
| Blocking | a = b | Inside combinational always block |
| Continuous | assign a = b | Wire-level combinational logic |

## 6.2 Reset Styles

- Synchronous: always @(posedge clk) — reset checked inside block with if (!rst_n)

- Asynchronous: always @(posedge clk or negedge rst_n) — reset fires immediately, not on next clock edge

- Both are valid for UART. Asynchronous is more common in real chips.

## 6.3 Common Pitfalls

- Using blocking (=) inside clocked block — causes simulation/synthesis mismatch

- Forgetting to initialise outputs in reset — leaves X (unknown) in simulation

- Counter bit width too small — wraps before reaching DIVISOR

- Missing default in case statement — causes latches in synthesis

- $clog2(N) gives exactly the right bit width for a counter that counts to N-1

# 7. Complete Signal Summary

| Signal | Direction | Module | Description |
| --- | --- | --- | --- |
| clk | input | all | System clock |
| rst_n | input | all | Active-low async reset |
| tick | output/input | brg→tx,rx | 16× baud pulse, 1 cycle wide |
| tx_start | input | uart_tx | 1-cycle pulse to begin TX |
| tx_data[7:0] | input | uart_tx | Byte to transmit (latch on tx_start) |
| tx_out | output | uart_tx | Serial TX line (idle=1) |
| tx_busy | output | uart_tx | High while transmitting |
| rx_in | input | uart_rx | Serial RX line |
| rx_data[7:0] | output | uart_rx | Received byte (valid when rx_valid=1) |
| rx_valid | output | uart_rx | 1-cycle pulse: rx_data is ready |

End of notes